I've done a little experimentation andhave come up with the connectionsrequired to construct a geoCable.  Thetable below shows the pin connectionsfor the User Port, the Centronicsconnector found on parallel interfaceprinters, and for the DB-25 connectorof the IBM-PC parallel port type.  Iinclude the last because the cable(Centronics to PC DB-25) is commonlyavailable.Instead of making a cable, I used acable for a PC and made an adaptor toplug it into the C64's User Port.  Theadaptor consists of a User Portconnector and a DB-25S glued to a smallpiece of circuit board, with wires runbetween the two connectors.Signal Name      PC DB-25             User Port       Centronics-STROBE      M       1       1   D0        C       2       2   D1        D       3       3   D2        E       4       4   D3        F       5       5   D4        H       6       6   D5        J       7       7   D6        K       8       8   D7        L       9       9  BUSY       B      11      11  GND    1,12,A,N 18-25  16,19-30,33                Ken WallaceCorrections made 17 May 89.##############################################