;	Commodore 128 I/O Map

;	V1.2	22 Jun 1994

COMMODORE 128 INPUT/OUTPUT ASSIGNMENTS

Differences Between C64 and C128

D500-D50B	MMU

D600-D601	VDC

FF00-FF05	Preconfiguration Registers

HEX	DECIMAL		BITS	DESCRIPTION

8502 I/O Registers

C128 mode:

0000	x	0	1	0	1	1	1	1

0001	x	Caps	Motor	Sense	Write	HiRes	LoRes	Color

C64 mode:

0000	0		7-0	MOS 8502 Data Direction

Register (xx101111)

Bit= 1: Output, Bit=0:

Input, x=Don't Care

0001	1			MOS 8502 Micro-Processor On-Chip I/O Port

0	/LORAM Signal (0=Switch	BASIC ROM Out)

1	/HIRAM Signal (0=Switch Kernal ROM Out)

2	/CHAREN Signal (O=Swith Char. ROM In)

3	Cassette Data Output Line

4	Cassette Switch Sense: 1 = Switch Closed

5	Cassette Motor Control

O = ON, 1 = OFF

6-7	Undefined

D000-D02E	53248-54271	MOS 6566 VIDEO INTERFACE CONTROLLER (VIC)

D000		53248		Sprite O X Pos

D001		53249		Sprite O Y Pos

D002		53250		Sprite 1 X Pos

D003		53251		Sprite 1 Y Pos

D004		53252		Sprite 2 X Pos

D005		53253		Sprite 2 Y Pos

D006		53254		Sprite 3 X Pos

D007		53255		Sprite 3 Y Pos

D008		53256		Sprite 4 X Pos

D009		53257		Sprite 4 Y Pos

D00A		53258		Sprite 5 X Pos

D00B		53259		Sprite 5 Y Pos

D00C		53260		Sprite 6 X Pos

D00D		53261		Sprite 6 Y Pos

D00E		53262		Sprite 7 X Pos

D00F		53263		Sprite 7 Y Pos

D010		53264		Sprites 0-7 X Pos (msb of X coord.)

D011		53265		VIC Control Register

7	Raster Compare: (Bit 8)	See 53266

6	Extended Color Text Mode 1 = Enable

5	Bit Map Mode. 1 = Enable

4	Blank Screen to Border Color: O = Blank

3	Select 24/25 Row Text Display: 1 = 25 Rows

2-0	Smooth Scroll to Y Dot-Position (0-7)

D012	53266			Read Raster / Write Raster Value for Compare IRQ

D013	53267			Light-Pen Latch X Pos

D014	53268			Light-Pen Latch Y Pos

D015	53269			Sprite display Enable: 1 = Enable

D016	53270			VIC Control Register

7-6	Unused

5	ALWAYS SET THIS BIT TO 0 !

4	Multi-Color Mode: 1 = Enable (Text or Bit-Map)

3	Select 38/40 Column Text Display: 1 = 40 Cols

2-0	Smooth Scroll to X Pos

D017	53271			Sprites O-7 Expand 2x Vertical (Y)

D018	53272			VIC Memory Control Register

7-4	Video Matrix Base Address (inside VIC)

3-1	Character Dot-Data Base	Address (inside VIC)

0	Select upper/lower Character Set

D019	53273			VIC Interrupt Flag Register (Bit = 1: IRQ Occurred)

7	Set on Any Enabled VIC IRQ Condition

3	Light-Pen Triggered IRQ Flag

2	Sprite to Sprite Collision IRQ Flag

1	Sprite to Background Collision IRQ Flag

0	Raster Compare IRQ Flag

D01A	53274			IRQ Mask Register: 1 = Interrupt Enabled

D01B	53275			Sprite to Background Display Priority: 1 = Sprite

D01C	53276			Sprites O-7 Multi-Color Mode Select: 1 = M.C.M.

D01D	53277			Sprites 0-7 Expand 2x Horizontal (X)

D01E	53278			Sprite to Sprite Collision Detect

D01F	53279			Sprite to Background Collision Detect

D020	53280			Border Color

D021	53281			Background Color O

D022	53282			Background Color 1

D023	53283			Background Color 2

D024	53284			Background Color 3

D025	53285			Sprite Multi-Color Register 0

D026	53286			Sprite Multi-Color Register 1

D027	53287			Sprite O Color

D028	53288			Sprite 1 Color

D029	53289			Sprite 2 Color

D02A	53290			Sprite 3 Color

D02B	53291			Sprite 4 Color

D02C	53292			Sprite 5 Color

D02D	53293			Sprite 6 Color

D02E	53294			Sprite 7 Color

D02F	53295		7-3	Unused

2-0	Additional Keyboard Columns

D030	53296		7-2	Unused

1	Test

0	2 MHz Mode

D400-D4FF	54272-52527	MOS 6581 SOUND INTERFACE DEVICE (SID)

D400	54272			Voice 1: Frequency Control - Low-Byte

D401	54273			Voice 1: Frequency Control - High-Byte

D402	54274			Voice 1: Pulse Waveform	Width - Low-Byte

D403	54275		7-4	Unused

3-0	Voice 1: Pulse Waveform Width - High-Nybble

D404	54276			Voice 1: Control Register

7	Select Random Noise Waveform, 1 = On

6	Select Pulse Waveform, 1 = On

5	Select Sawtooth Waveform, 1 = On

4	Select Triangle Waveform, 1 = On

3	Test Bit: 1 = Disable Oscillator 1

2	Ring Modulate Osc. 1 with Osc. 3 Output, 1 = On

1	Synchronize Osc. 1 with Osc. 3 Frequency, 1 = On

0	Gate Bit: 1 = Start Att/Dec/Sus, 0 = Start Release

D405	54277			Envelope Generator 1: Attack / Decay Cycle Control

7-4	Select Attack Cycle Duration: O-15

3-0	Select Decay Cycle Duration: 0-15

D406	54278			Envelope Generator 1: Sustain / Release Cycle Control

7-4	Select Sustain Cycle Duration: O-15

3-0	Select Release Cycle Duration: O-15

D407	54279			Voice 2: Frequency Control - Low-Byte

D408	54280			Voice 2: Frequency Control - High-Byte

D409	54281			Voice 2: Pulse Waveform Width - Low-Byte

D40A	54282		7-4	Unused

3-0	Voice 2: Pulse Waveform Width - High-Nybble

D40B	54283			Voice 2: Control Register

7	Select Random Noise Waveform, 1 = On

6	Select Pulse Waveform, 1 = On

5	Select Sawtooth Waveform, 1 = On

4	Select Triangle Waveform, 1 = On

3	Test Bit: 1 = Disable Oscillator 1

2	Ring Modulate Osc. 2 with Osc. 1 Output, 1 = On

1	Synchronize Osc. 2 with Osc. 1 Frequency, 1 = On

0	Gate Bit: 1 = Start Att/Dec/Sus, 0 = Start Release

D40C	54284			Envelope Generator 2: Attack / Decay Cycle Control

7-4	Select Attack Cycle Duration: O-15

3-0	Select Decay Cycle Duration: 0-15

D40D	54285			Envelope Generator 2: Sustain / Release Cycle Control

7-4	Select Sustain Cycle Duration: O-15

3-0	Select Release Cycle Duration: O-15

D40E	54286			Voice 3: Frequency Control - Low-Byte

D40F	54287			Voice 3: Frequency Control - High-Byte

D410	54288			Voice 3: Pulse Waveform Width - Low-Byte

D411	54289		7-4	Unused

3-0	Voice 3: Pulse Waveform Width - High-Nybble

D412	54290			Voice 3: Control Register

7	Select Random Noise Waveform, 1 = On

6	Select Pulse Waveform, 1 = On

5	Select Sawtooth Waveform, 1 = On

4	Select Triangle Waveform, 1 = On

3	Test Bit: 1 = Disable Oscillator 1

2	Ring Modulate Osc. 3 with Osc. 2 Output, 1 = On

1	Synchronize Osc. 3 with Osc. 2 Frequency, 1 = On

0	Gate Bit: 1 = Start Att/Dec/Sus, 0 = Start Release

D413	54291		Envelope Generator 3: Attac/Decay Cycle Control

7-4	Select Attack Cycle Duration: O-15

3-0	Select Decay Cycle Duration: 0-15

D414	54285			Envelope Generator 3: Sustain / Release Cycle Control

7-4	Select Sustain Cycle Duration: O-15

3-0	Select Release Cycle Duration: O-15

D415	54293			Filter Cutoff Frequency: Low-Nybble (Bits 2-O)

D416	54294			Filter Cutoff Frequency: High-Byte

D417	54295			Filter Resonance Control / Voice Input Control

7-4	Select Filter Resonance: 0-15

3	Filter External Input: 1 = Yes, 0 = No

2	Filter Voice 3 Output: 1 = Yes, 0 = No

Filter Voice 2 Output: 1 = Yes, 0 = No

0	Filter Voice 1 Output: 1 = Yes, 0 = No

D418	54296			Select Filter Mode and Volume

7	Cut-Off Voice 3 Output: 1 = Off, O = On

6	Select Filter High-Pass Mode: 1 = On

5	Select Filter Band-Pass Mode: 1 = On

4	Select Filter Low-Pass Mode: 1 = On

3-0	Select Output Volume: 0-15

D419	54297			Analog/Digital Converter: Game Paddle 1 (O-255)

D41A	54298			Analog/Digital Converter Game Paddle 2 (O-255)

D41B	54299			Oscillator 3 Random Number Generator

D41C	54230			Envelope Generator 3 Output

MMU  $D500 and $FF00

C128 MMU  8722

D500		Configuration Register (CR)

7-6	RAM-select 0-3

5-4	High RAM/ROM

00 System ROM (Kernel, Edit)

01 Internal Function ROM

10 External Function ROM

11 RAM

NOTE: I/O overrides all of these.

3-2	Mid RAM/ROM

00 System ROM (Basic HI)

01 Internal Function ROM

10 External Function ROM

11 RAM

1	Lo RAM

0  System ROM (Basic LO)

1  RAM

0	C.GEN

0  I/O

1  ROM/RAM

D501		Preconfiguration Registers

D502

D503

D504

D505	7	40/80 read only

6	C64 Mode (0 = C128)

5	EXROM: C64 (1 = C128) read only

4	GAME:  C64 (1 = C128) read only

3	FSDIR: Fast Disk Transfer Direction

2-1	-

0	Z80/8502

D506		RAM Configuration Register (RCR)

7-6	Video-Bank

5-4	-

3	Shared RAM Hi

2	Shared RAM Lo

1-0	Shared RAM Size: 00 = 1K, 01 = 4K, 10 = 8K, 11 = 16K

D507		Zero Page Pointer Lo

7-0	A15-A8

D508		Zero Page Pointer Hi

7-4	-

3-2	A19-A18 (Used in 1MB System)

1-0	A17-A16	(256K System)

D509		Stack Page Pointer Lo

D50A		Stack Page Pointer Hi

D50B		MMU Version Register

7-4	Bank Version (2 = 128K)

3-0	MMU Version

Default Memory Configurations

Bank   FF00    Free RAM Space	RAM I/O  ROM

0	3f	0000-ffff	 0

1	7f	0000-ffff	 1

2	bf	0000-ffff	 2

3	ff	0000-ffff	 3

4	16	0000-7fff	 0  I/O  Int

5	56	0000-7fff	 1  I/O  Int

6	96	0000-7fff	 2  I/O  Int

7	d6	0000-7fff	 3  I/O  Int

8	2a	0000-7fff	 0  I/O  Ext

9	6a	0000-7fff	 1  I/O  Ext

10	aa	0000-7fff	 2  I/O  Ext

11	ea	0000-7fff	 3  I/O  Ext

12	06	0000-7fff	 0  I/O  Kernel Int_Rom_Low

13	0a	0000-7fff	 0  I/O  Kernel Ext_Rom_Low

14	01	0000-3fff	 0  Char Kernel Basic

15	00	0000-3fff	 0  I/O  Kernel Basic

VDC  $D600

This register map is from C-Hacking Magazine.

----------------------------------

| VDC 8563  Register Definitions |

----------------------------------

Reg     7    6    5    4    3    2    1    0     Description              Notes

------ ---- ---- ---- ---- ---- ---- ---- ----   ------------------------ -----

0	HzT7 HzT6 HzT5 HzT4 HzT3 HzT2 HzT1 HzT0   Horizontal Total	    ^1

1	HzD7 HzD6 HzD5 HzD4 HzD3 HzD2 HzD1 HzD0   Horizontal Displayed      ^1

2	HzS7 HzS6 HzS5 HzS4 HzS3 HzS2 HzS2 HzS0   Horizontal Sync Position  ^1

3	VSW3 VSW2 VSW1 VSW0 HSW3 HSW2 HSW1 HSW0   Vert/Horiz. Sync Width    ^2

4	VeT7 VeT6 VeT5 VeT4 VeT3 VeT2 VeT1 VeT0   Vertical Total	    ^3

5	.... .... .... VeA4 VeA3 VeA2 VeA1 VeA0   Vertical Total Fine Adju  ^3

6	VeD7 VeD6 VeD5 VeD4 VeD3 VeD2 VeD1 VeD0   Vertical Displayed	    ^3

7	VeS7 VeS6 VeS5 VeS4 VeS3 VeS2 VeS1 VeS0   Vertical Sync Position    ^2

8	.... .... .... .... .... .... Ilc1 Ilc0   Interlace Mode            ^4

9	.... .... .... CTV4 CTV3 CTV2 CTV1 CTV0   Character Total Vertical  ^5

0a  10	.... CrM1 CrM0 Css4 Css3 Css2 Css1 Css0   Cursor Mode/ Start Scan   ^6

0b  11	.... .... .... Ces4 Ces3 Ces2 Ces1 Ces0   Cursor End Scan    	    ^6

0c  12	Ds15 Ds14 Ds13 Ds12 Ds11 Ds10 Ds09 Ds08   Display Start Adrs (Hi)   ^7

0d  13	Ds07 Ds06 Ds05 Ds04 Ds03 Ds02 Ds01 Ds00   Display Start Adrs (Lo)   ^7

0e  14	Cp15 Cp14 Cp13 Cp12 Cp11 Cp10 Cp09 Cp08   Cursor Position (Hi)	    ^7

0f  15	Cp07 Cp06 Cp05 Cp04 Cp03 Cp02 Cp01 Cp00   Cursor Position (Lo)	    ^7

10  16	LpV7 LpV6 LpV5 LpV4 LpV3 LpV2 LpV1 LpV0   Light Pen Veritcal	    ^8

11  17	LpH7 LpH6 LpH5 LpH4 LpH3 LpH2 LpH1 LpH0   Light Pen Horizontal	    ^8

12  18	Ua15 Ua14 Ua13 Ua12 Ua11 Ua10 Ua09 Ua08   Update Address (Hi)	    ^9

13  19	Ua07 Ua06 Ua05 Ua04 Ua03 Ua02 Ua01 Ua00   Update Address (Lo)	    ^9

14  20	At15 At14 At13 At12 At11 At10 At09 At08   Attribute Start Adrs (Hi) ^7

15  21	At07 At06 At05 At04 At03 At02 At01 At00   Attribute Start Adrs (Lo) ^7

16  22	HcP3 HcP2 HcP1 HcP0 IcS3 IcS2 IcS1 IcS0   Hz Chr Pxl Ttl/IChar Spc  ^A

17  23	.... .... .... VcP4 VcP3 VcP2 VcP1 VcP0   Vert. Character Pxl Spc   ^5

18  24	BlkM RvsS Vss5 Vss4 Vss3 Vss2 Vss1 Vss0   Block/Rvs Scr/V. Scroll ^9^B^C

19  25	Text Atri Semi Dble Hss3 Hss2 Hss1 Hss0   Diff. Mode Sw/H. Scroll  ^D,^E

1a  26	Fgd3 Fgd2 Fgd1 Fgd0 Bgd3 Bgd2 Bgd1 Bgd0   ForeGround/BackGround Col ^F

1b  27	Rin7 Rin6 Rin5 Rin4 Rin3 Rin2 Rin1 Rin0   Row/Adrs. Increment	    ^G

1c  28	CSa2 CSa1 CSa0 RamT .... .... .... ....   Character Set Addrs/Ram   ^H,^I

1d  29	.... .... .... UdL4 UdL3 UdL2 UdL1 UdL0   Underline Scan Line	    ^6

1e  30	WdC7 WdC6 WdC5 WdC4 WdC3 WdC2 WdC1 WdC0   Word Count (-1)	    ^9

1f  31	Dta7 Dta6 Dta5 Dta4 Dta3 Dta2 Dta1 Dta0   Data			    ^9

20  32	BlkF BlkE BlkD BlkC BlkB BlkA Blk9 Blk8   Block Copy Source (hi)    ^9

21  33	Blk7 Blk6 Blk5 Blk4 Blk3 Blk2 Blk1 Blk0   Block Copy Source (lo)    ^9

22  34	DeB7 DeB6 DeB5 DeB4 DeB3 DeB2 DeB1 DeB0   Display Enable Begin	    ^J

23  35	DeE7 DeE6 DeE5 DeE4 DeE3 DeE2 DeE1 DeE0   Display Enable End	    ^J

24  36	.... .... .... .... Drm3 Drm2 Drm1 Drm0   DRAM Refresh Rate	    ^K

D800-DBFF	55296-56319	Color RAM (Nybbles)

DC00-DCFF	56320-56575	MOS 6526 Complex Interface Adapter (CIA) #1

DC00	56320			Data Port A (Keyboard, Joystick, Paddles, Light-Pen)

7-0	Write Keyboard Column Values for Keyboard Scan

7-6	Read Paddles on Port A / B (01 = Port A, 10 = Port B)

4	Joystick A Fire Button: 1 = Fire

3-2	Paddle Fire Buttons

3-0	Joystick A Direction (0-15)

DC01	56321			Data Port B (Keyboard, Joystick, Paddles): Game Port 1

7-0	Read Keyboard Row Values for Keyboard Scan

7	Timer B Toggle/Pulse Output

6	Timer A: Toggle/Pulse Output

4	Joystick 1 Fire Button: 1 = Fire

3-2	Paddle Fire Buttons

3-0	Joystick 1 Direction

DC02	56322			Data Direction Register - Port A (56320)

DC03	56323			Data Direction Register - Port B (56321)

DC04	56324			Timer A: Low-Byte

DC05	56325			Timer A: High-Byte

DC06	56326			Timer B: Low-Byte

DC07	56327			Timer B: High-Byte

DC08	56328			Time-of-Day Clock: 1/10 Seconds

DC09	56329			Time-of-Day Clock: Seconds

DC0A	56330			Time-of-Day Clock: Minutes

DC0B	56331			Time-of-Day Clock: Hours + AM/PM Flag (Bit 7)

DC0C	56332			Synchronous Serial I/O Data Buffer

DC0D	56333			CIA Interrupt Control Register (Read IRQs/Write Mask)

7	IRQ Flag (1 = IRQ Occurred) / Set-Clear Flag

4	FLAG1 IRQ (Cassette Read / Serial Bus SRQ Input)

3	Serial Port Interrupt

2	Time-of-Day Clock Alarm Interrupt

1	Timer B Interrupt

0	Timer A Interrupt

DC0E	56334			CIA Control Register A

7	Time-of-Day Clock Frequency: 1 = 50 Hz, 0 = 60 Hz

6	Serial Port I/O Mode Output, 0 = Input

5	Timer A Counts: 1 = CNT Signals, 0 = System 02 Clock

4	Force Load Timer A: 1 = Yes

3	Timer A Run Mode: 1 = One-Shot, 0 = Continuous

2	Timer A Output Mode to PB6: 1 = Toggle, 0 = Pulse

1	Timer A Output on PB6: 1 = Yes, 0 = No

0	Start/Stop Timer A: 1 = Start, 0 = Stop

DC0F	56335			CIA Control Register B

7	Set Alarm/TOD-Clock: 1 = Alarm, 0 = Clock

6-5	Timer B Mode Select:

00 = Count System 02 Clock Pulses

01 = Count Positive CNT Transitions

10 = Count Timer A Underflow Pulses

11 = Count Timer A Underflows While CNT Positive

4-0	Same as CIA Control Reg. A - for Timer B

DD00-DDFF	56576-56831		MOS 6526 Complex Interface Adapter (CIA) #2

DD00	56576			Data Port A (Serial Bus, RS-232, VIC Memory Control)

7	Serial Bus Data Input

6	Serial Bus Clock Pulse Input

5	Serial Bus Data Output

4	Serial Bus Clock Pulse Output

3	Serial Bus ATN Signal Output

2	RS-232 Data Output (User Port)

1-O	VIC Chip System Memory Bank Select (Default = 11)

DD01	56577		Data Port B (User Port, RS-232)

7	User / RS-232 Data Set Ready

6	User / RS-232 Clear to Send

5	User

4	User / RS-232 Carrier Detect

3	User / RS-232 Ring Indicator

2	User / RS-232 Data Terminal Ready

1	User / RS-232 Request to Send

0	User / RS-232 Received Data

DD02	56578			Data Direction Register - Port A

DD03	56579			Data Direction Register - Port B

DD04	56580			Timer A: Low-Byte

DD05	56581			Timer A: High-Byte

DD06	56582			Timer B: Low-Byte

DD07	56583			Timer B: High-Byte

DD08	56584			Time-of-Day Clock: 1/10 Seconds

DD09	56585			Time-of-Day Clock: Seconds

DD0A	56586			Time-of-Day Clock: Minutes

DD0B	56587			Time-of-Day Clock: Hours + AM/PM Flag (Bit 7)

DD0C	56588			Synchronous Serial I/O Data Buffer

DD0D	56589			CIA Interrupt Control Register (Read NMls/Write Mask)

7	NMI Flag (1 = NMI Occurred) / Set-Clear Flag

4	FLAG1 NMI (User/RS-232 Received Data Input)

3	Serial Port Interrupt

1	Timer B Interrupt

0	Timer A Interrupt

DD0E	56590			CIA Control Register A

7	Time-of-Day Clock Frequency: 1 = 50 Hz, 0 = 60 Hz

6	Serial Port I/O Mode Output, 0 = Input

5	Timer A Counts: 1 = CNT Signals, 0 = System 02 Clock

4	Force Load Timer A: 1 = Yes

3	Timer A Run Mode: 1 = One-Shot, 0 = Continuous

2	Timer A Output Mode to PB6: 1 = Toggle, 0 = Pulse

1	Timer A Output on PB6: 1 = Yes, 0 = No

0	Start/Stop Timer A: 1 = Start, 0 = Stop

DD0F	56591			CIA Control Register B

7	Set Alarm/TOD-Clock: 1 = Alarm, 0 = Clock

6-5	Timer B Mode Select:

00 = Count System 02 Clock Pulses

01 = Count Positive CNT Transitions

10 = Count Timer A Underflow Pulses

11 = Count Timer A Underflows While CNT Positive

4-0	Same as CIA Control Reg. A - for Timer B

DEOO-DEFF	56832-57087	Reserved for Future I/O Expansion

DFOO-DFFF	57088-57343	Reserved for Future I/O Expansion

DMA  $DF00

8726 DMA Controller for C128 (512K REU)

Note: The 8726 DMA controller for C128 is different from the C64 ones

in registers 00,01,09 and 0a.

DF00  DMA ST	STATUS

7	Interrupt Pending (1 = Int. waiting)

6	End of Block (1 = Transfer complete)

5	Fault  (1 = Block verify error)

4	Size   (0 = 128K, 1 = 512K Exp. Memory)

3-0	Version

DF01  DMA CMD	COMMAND

7	Exec

6	Reserved (normally 0)

5	Load  (1 = Enable auto load)

4	$FF00 (1 = Disable $FF00 decades)

3-2	Reserved (normally 0)

1-0	Mode

00 = Transfer from internal to external

01 = From ext to int

10 = Swap

11 = Verify

DF02  DMA ADL	HOST ADDRESS LOW

DF03  DMA ADH	HOST ADDRESS HIGH

DF04  DMA LO	EXPANSION ADDRESS LOW

DF05  DMA HI	EXPANSION ADDRESS HIGH

DF06  DMA BNK	EXPANSION BANK (bits 2-0 only)

7-3	Unused

2-0	Expansion Bank number

DF07  DMA DAL	TRANSFER LENGTH LOW

DF08  DMA DAH	TRANSFER LENGTH HIGH

DF09  DMA SUM	INTERRUPT MASK REGISTER

7	Interrupt Enable (1 = Interrupts enabled)

6	End of Block mask (1 = Interrupt on end of block)

5	Verify Error (1 = Interrupt on verify error)

4-0	Unused (normally all set)

DF0A  DMA VER	VERSION, MAXIMUM MEMORY

7-6	Address Control

00 = Increment both addresses (default)

01 = Fix expansion address

10 = Fix C128 address

11 = Fix both addresses

5-0	Unused (normally all set)

####################################################