ADRAY2	0005-0006	5-6	Jump Vector: Convert Integer--Floating

CHARAC	0007		7	Search Character

ENDCHR	0008		8	Flag: Scan for Quote at End of String

TRMPOS	0009		9	Screen Column From Last TAB

VERCK	000A		10	Flag: 0 = Load, 1 = Verify

COUNT	000B		11	Input Buffer Pointer / No. of Subscripts

DIMFLG	000C		12	Flag: Default Array DiMension

VAUYP	000D		13	Data Type: $FF = String, $00 = Numeric

INTFLG	000E		14	Data Type: $80 = Integer, $00 = Floating

GARBFL	000F		15	Flag: DATA scan/LlST quote/Garbage Coll

SUBFLG	0010		16	Flag: Subscript Ref / User Function Call

INPFLG	0011		17	Flag: $00 = INPUT, $40 = GET, $98 = READ

TANSGN	0012		18	Flag TAN sign / Comparison Result

0013		19	Flag: INPUT Prompt

LINNUM	0014-0015	20-21	Temp: Integer Value

TEMPPT	0016		22	Pointer Temporary String

LASTPT	0017-0018	23-24	Last Temp String Address

TEMPST	0019-0021	25-33	Stack for Temporary Strings

INDEX	0022-002S	34-37	Utility Pointer Area

RESHO	0026-002A	38-42	Floating-Point Product of Multiply

TXTTA3	002B-002C	43-44	Pointer: Start of BASIC	Text

BASIC TO MACHINE LANGUAGE	311

LABEL	ADDRESS		DEC	DESCRIPTION

VARTAB	002D-002E	45-46	Pointer: Start of BASIC	Variables

ARYTAB	002F-0030	47-48	Pointer: Start of BASIC Arrays

STREND	0031-0032	49-50	Pointer End of BASIC Arrays (+1)

FRETOP	0033-0034	51-52	Pointer: Bottom of String Storage

FRESPC	0035-0036	53-54	Utility String Pointer

MEMSIZ	0037-0038	55-56	Pointer: Highest Address Used by BASIC

CURLIN	0039-003A	57-58	Current BASIC Line Number

OLDLIN	003B-003C	59-60	Previous BASIC Line Number

OLDTXT	003D-003E	61-62	Pointer: BASIC Statement for CONT

DATLIN	003F-0040	63-64	Current DATA Line Number

DATPTR	0041-0042	65-66	Pointer: Current DATA Item Address

INPPTR	0043-0044	67-68	Vector: INPUT Routine

VARNAM	0045-0046	69-70	Current BASIC Variable Name

VARPNT	0047-0048	71-72	Pointer: Current BASIC Variable Data

FORPNT	0049-004A	73-74	Pointer: Index Variable for FOR/NEXT

004B-0060	75-96	Temp Pointer / Data Area

FACEXP	0061		97	Floating-Point Accumulator #1: Exponent

FACHO	0062-0065	98-101	Floating Accum.	#1: Mantissa

FACSGN	0066		102	Floating Accum.	#1: Sign

SGNFLG	0067		103	Pointer: Series Evaluation Constant

312 BASIC TO MACHINE LANGUAGE

DEC	DESCRIPTION

BITS	0068		104	Floating -accum. #1: Overflow Digit

ARGEXP	0069		105	Floating-Point Accumulator #2: Exponent

ARGHO	006A-006D	106-109	Floating Accum. #2: Mantissa

ARGSGN	006E		110	Floating Accum. #2: Sign

ARISGN	006F		111	Sign Comparison Result: Accum. # 1 vs #2

FACOV	0070		112	Floating Accum. #1. Low-Order (Rounding)

FBUFPT	0071-0072	113-114	Pointer: Cassette Buffer

CHRGET	0073-008A	115-138	Subroutine: Get Next Byte of BASIC Text

CHRGOT	0079		121	Entry to Get Same Byte of Text Again

TXTPTR	007A-007B	122-123	Pointer: Current Byte of BASIC Text

RNDX	008B-008F	139-143	Floating RND Function Seed Value

STATUS	0090		144	Kernal l/O Status Word: ST

STKEY	0091		145	Flag: STOP key / RVS key

SVXT	0092		146	Timing Constant for Tape

VERCK	0093		147	Flag: 0 = Load, 1 = Verify

C3PO	0094		148	Flag: Serial Bus-Output Char. Buffered

BSOUR	0095		149	Buffered Character for Serial Bus

SYNO	0096		150	Cassette Sync No.

0097		151	Temp Data Area

LDTND	0098		152	No. of Open Files / Index to File Table

DFLTN	0099		153	Default Input Device (0)

DFLTO	009A		154	Default Output (CMD) Device (3)

BASIC TO MACHINE LANGUAGE	313

LABEL	ADDRESS		LOCATION DESCRLPTLON

PRTY	009B		155	Tape Character Parity

PSW	009C		156	Flag: Tape Byte-Received

MSGFLG	009D		157	Flag: $80 = Direct Mode, $00 = Program

PTR1	009E		158	Tape Pass 1 Error Log

PTR2	009F		159	Tape Pass 2 Error Log

TIME	00A0-00A2	160-162	Real-Time Jiffy Clock (approx) 1/60 Sec

00A3-00A4	163-164	Temp Data Area

CNTDN	00A5		165	Cassette Sync Countdown

BUFPNT	00A6		166	Pointer: Tape l/O Buffer

INBIT	00A7		167	RS-232 Input Bits / Cassette Temp

BITCI	00A8		168	RS-232 Input Bit Count / Cassette Temp

RINONE	00A9		169	RS-232 Flag: Check for Start Bit

RIDATA	00AA		170	RS-232 Input Byte Buffer/Cassette Temp

RIPRTY	00AB		171	RS-232 Input Parity / Cassette Short Cnt

SAL	00AC-OOAD	172-173	Pointer: Tape Buffer/ Screen Scrolling

EAL	00AE-00AF	174-175	Tape End Addresses/End of Program

CMP0	00B0-00B1	176-177	Tape Timing Constants

TAPE1	00B2-00B3	178-179	Pointer: Start of Tape Buffer

BITTS	00B4		180	RS-232 Out Bit Count / Cassette Temp

NXTBIT	00B5		181	RS-232 Next Bit to Send/ Tape EOT Flag

RODATA	00B6		182	RS-232 Out Byte Buffer

FNLEN	00B7		183	Length of Current File Name

LA	00B8		184	Current Logical File Number

314	BASIC TO MACHINE LANGUAGE

FA	00BA		186	Current Device Number

FNADR	00BB-00BC	187-188	Pointer: Current File Name

ROPRTY	00BD		189	RS-232 Out Parity / Cassette Temp

FSBLK	00BE		190	Cassette Read / Write Block Count

MYCH	00BF		191	Serial Word Buffer

CAS1	00C0		192	Tape Motor Interlock

STAL	00C1-00C2	193-194	I/O Start Address

MEMUSS	00C3-00C4	195-196	Tape Load Temps

LSTX	00C5		197	Current Key Pressed: CHR$(n) 0 = No Key

NDX	00C6		198	No. of Chars. in Keyboard Buffer (Queue)

RVS	00C7		199	Flag: Print Reverse Chars. -1=Yes, 0=No Used

INDX	00C8		200	Pointer: End of Logical	Line for INPUT

LXSP	00C9-00CA	201-202	Cursor X-Y Pos. at Start of INPUT

SFDX	00CB		203	Flag: Print Shifted Chars.

BLNSW	00CC		204	Cursor Blink enable: 0 = Flash Cursor

BLNCT	00CD		205	Timer: Countdown to Toggle Cursor

GDBLN	00CE		206	Character Under Cursor

BLNON	00CF		207	Flag: Last Cursor Blink	On/Off

CRSW	00D0		208	Flag: INPUT or GET from	Keyboard

PNT	00D1-00D2	209-210	Pointer: Current Screen	Line Address

BASIC TO MACHINE LANGUAGE	315

HEX		DECIMAL

LABEL	ADDRESS	LOCATION	 DESCRIPTION

PNTR	00D3		211	Cursor Column on Current Line

QTSW	00D4		212	Flag: Editor in Quote Mode, $00 = NO

LNMX	00D5		213	Physical Screen Line Length

TBLX	00D6		214	Current Cursor Physical Line Number

00D7		215	Temp Data Area

INSRT	00D8		216	Flag: Insert Mode, >O = # INSTs

LDTB1	00D9-00F2	217-242	Screen Line Link Table / Editor Temps

USER	00F3-00F4	243-244	Pointer: Current Screen	Color RAM loc.

KEYTAB	00F5-00F6	24S-246	Vector Keyboard Decode Table

RIBUF	00F7-00F8	247-248	RS-232 Input Buffer Pointer

ROBUF	00F9-00FA	249-250	RS-232 Output Buffer  Pointer

FREKZP	00FB-00FE	251-254	Free 0-Page Space for User Programs

BASZPT	00FF		255	BASIC Temp Data Area

0100-01FF	256-511	Micro-Processor System Stack Area

0100-010A	256-266	Floating to String Work Area

BAD	0100-013E	256-318	Tape Input Error Log

BUF	0200-02S8	512-600	System INPUT Buffer

LAT	0259-0262	601-610	KERNAL Table: Active Logical File No's.

FAT	0263-026C	611-620	KERNAL Table: Device No. for Each File

SAT	026D-0276	621-630	KERNAL Table: Second Address Each File

KEYD	0277-0280	631-640	Keyboard Buffer Queue (FIFO)

316	 BASIC TO MACHlNE LANGUAGE

MEMSTR	0281-0282	641-642	Pointer: Bottom of Memory for O.S.

MEMSIZ	0283-0284	643-644	Pointer: Top of Memory for O.S.

TIMOUT	0285		645	Flag: Kernal Variable for IEEE Timeout

COLOR	0286		646	Current Character Color Code

GDCOL	0287		647	Background Color Under Cursor

HIBASE	0288		648	Top of Screen Memory (Page)

XMAX	0289		649	Size of Keyboard Buffer

RPTFLG	028A		650	Flag: REPEAT Key Used, $80 = Repeat

KOUNT	028B		651	Repeat Speed Counter

DELAY	028C		652	Repeat Delay Counter

SHFLAG	028D		653	Flag: Keyb'rd SHIFT Key/CTRL Key/C= Key

LSTSHF	028E		654	Last Keyboard Shift Pattern

KEYLOG	028F-0290	655-656	Vector: Keyboard Table Setup

MODE	0291		657	Flag: $00=Disable SHIFT	Keys, $80 = Enable

SHIFT Keys

AUTODN	0292		658	Flag: Auto Scroll Down, 0 = ON

M51CTR	0293		659	RS-232: 6551 Control Register Image

MS1CDR	0294		660	RS-232: 6551 Command Register Image

M51AJB	0295-0296	661-662	RS-232 Non-Standard BPS (Time/2-100) USA

RSSTAT	0297		663	RS-232: 6551 Status Register Image

BITNUM	0298		664	RS-232 Number of Bits Left to Send

BASIC TO MACHINE LANGUAGE	317

LABEL	HEX		DECIMAL

ADDRESS		LOCATION	DESCRIPTION

BAUDOF	0299-029A	665-666	RS-232 Baud Rate: Full Bit Time (us)

RIDBE	029B		667	RS-232 Index to End of Input Buffer

RIDBS	029C		668	RS-232 Start of Input Buffer (Page)

RODBS	029D		669	RS-232 Start of Output Buffer (Page)

RODBE	029E		670	RS-232 Index to End of Output Buffer

IRQTMP	029F-02A0	671-672	Holds IRQ Vector During Tape I/O

ENABL	02A1		673	RS-232 Enables

02A2		674	TOD Sense During Cassette I/O

02A3		675	Temp Storage For Cassette Read

02A4		676	Temp D1 IRQ Indicator For Cassette Read

02A5		677	Temp For Line Index

02A6		678	PAL/NTSC Flag, O= NTSC, 1 = PAL

02A7-02FF	679-767	Unused

IERROR	0300-0301	768-769	Vector: Print BASIC Error Message

IMAIN	0302-0303	770-771	Vector: BASIC Warm Start

ICRNCH	0304-0305	772-773	Vector: Tokenize BASIC Text

IQPLOP	0306-0307	774-775	Vector: BASIC Text LIST

IGONE	0308-0309	776-777	Vector: BASIC Char. Dispatch

IEVAL	030A-030B	778-779	Vector: BASIC Token Evaluation

SAREG	030C		780	Storage for 6502 .A Register

SXREG	030D		781	Storage for 5502 .X Register

318	BASIC TO MACHINE LANGUAGE

LABEL	ADDRESS		LOCATION		DESC

SYREG	030E		782	Storage for 6502 .Y Register

SPREG	030F		783	Storage for 6502 .SP Register

USRPOK	0310		784	USR Function Jump Instr (4C)

USRADD	0311-0312	785-786	USR Address Low	Byte / High Byte

0313		787	Unused

CINV	0314-0315	788-789	Vector: Hardware Interrupt

CBINV	0316-0317	790-791	Vector: BRK Instr. Interrupt

NMINV	0318-0319	792-793	Vector: Non-Maskable Interrupt

IOPEN	031A-031B	794-795	KERNAL OPEN Routine Vector

ICLOSE	031C-031D	796-797	KERNAL CLOSE Routine Vector

ICHKIN	031E-031F	798-799	KERNAL CHKIN Routine

ICKOUT	0320-0321	800-801	KERNAL CHKOUT Routine

ICLRCH	0322-0323	802-803	KERNAL CLRCHN Routine Vector

IBASIN	0324-0325	804-805	KERNAL CHRIN Routine

IBSOUT	0326-0327	806-807	KERNAL CHROUT Routine

ISTOP	0328-0329	808-809	KERNAL STOP Routine Vector

IGETIN	032A-032B	810-811	KERNAL GETIN Routine

ICLALL	032C-032D	812-813	KERNAL CLALL Routine Vector

USRCMD	032E-032F	814-815	User-Defined Vector

ILOAD	0330 0331	813 817	KERNAL LOAD Routine

BASIC TO MACHINE LANGUAGE	319

LABEL	ADDRESS

ISAVE	0332-0333	818-819		KERNAL SAVE Routine Vector

0334-033B	820-827		Unused

TBUFFR	033C-03FB	828-1019	Tape l/O Buffer

03FC-03FF	1020-1023	Unused

VICSCN	0400-07FF	1024-2047	1024 Byte Screen Memory Area

0400-07E7	1024-2023	Video Matrix: 25 Lines X 40 Columns

07F8-07FF	2040-2047	Sprite Data Pointers

0800-9FFF	2048-40959	Normal BASIC Program Space

8000-9FFF	132768-40959	VSP Cartridge ROM -- 8192 Bytes

A000-BFFF	40960-49151	BASIC ROM-8192 Bytes (or 8K RAM)

C000-CFFF	49152-53247	RAM-4096 Bytes

D000-DFFF	53248-57343	Input/Output Devices and

Color RAM or Character Generator ROM

or RAM-4096 Bytes

E000-FFFF	57344-65535	KERNAL ROM -- 8192 Bytes (or 8K RAM)

#############################################################