- 1 -                     XMODEM/YMODEM PROTOCOL REFERENCE                 A compendium of documents describing the                            XMODEM and YMODEM                         File Transfer Protocols                   This document was formatted 9-11-86.                         Edited by Chuck Forsberg                 Please distribute as widely as possible.                       Questions to Chuck Forsberg                           Omen Technology Inc                        17505-V Sauvie Island Road                          Portland Oregon 97231                           Voice: 503-621-3406            Modem (Telegodzilla): 503-621-3746 Speed 1200,300                          Compuserve: 70007,2304                    UUCP: ...!tektronix!reed!omen!caf                                  - 2 -1.  ROSETTA STONEHere are some definitions which reflect the current vernacular in thecomputer media.  The attempt here is identify the file transfer protocolrather than specific programs.XMODEM  refers to the original 1979 file transfer etiquette introduced by        Ward Christensen's 1979 MODEM2 program.  It's also called the        MODEM or MODEM2 protocol.  Some who are unaware of MODEM7's        unusual batch file mode call it MODEM7.  Other aliases include        "CP/M Users's Group" and "TERM II FTP 3".  This protocol is        supported by every serious communications program because of its        universality, simplicity, and reasonable performance.XMODEM/CRC replaces XMODEM's 1 byte checksum with a two byte Cyclical        Redundancy Check (CRC-16), giving modern error detection        protection.XMODEM-1k Refers to the XMODEM/CRC protocol with 1024 byte data blocks.YMODEM  refers to the XMODEM/CRC (optional 1k blocks) protocol with the        batch transmission described below.ZMODEM  uses familiar XMODEM/CRC and YMODEM technology in a new protocol        that provides reliability, throughput, file management, and user        amenities appropriate to contemporary data communications.2.  YET ANOTHER PROTOCOL?Since its development half a decade ago, the Ward Christensen modemprotocol has enabled a wide variety of computer systems to interchangedata.  There is hardly a communications program that doesn't at leastclaim to support this protocol.Recent advances in computing, modems and networking have revealed a numberof weaknesses in the original protocol:   + The short block length caused throughput to suffer when used with     timesharing systems, packet switched networks, satellite circuits,     and bufffirst      to the sending program and then again to the     receiving program.   + The transmitted file could accumulate as many as 127 extraneous     bytes.Chapter 2X/YMODEM Protocol Reference      09-11-86                                3   + The modification date of the file was lost.A number of other protocols have been developed over the years, but nonehave displaced XMODEM to date:   + Lack of public domain documentation and example programs have kept     proprietary protocols such as MNP, Blast, and others tightly bound to     the fortunes of their suppliers.   + Complexity discourages the widespread application of BISYNC, SDLC,     HDLC, X.25, and X.PC protocols.   + Performance compromises and moderate complexity have limited the     popularity of the Kermit protocol, which was developed to allow file     transfers in environments hostile to XMODEM.The XMODEM protocol extensions and YMODEM Batch address these weaknesseswhile maintaining XMODEM's simplicity.YMODEM is supported by the public domain programs YAM (CP/M),YAM(CP/M-86), YAM(CCPM-86), IMP (CP/M), KMD (CP/M), rz/sz (Unix, Xenix,VMS, Berkeley Unix, Venix, Xenix, Coherent, IDRIS, Regulus).  Commericalimplementations include MIRROR, and Professional-YAM.[1] Communicationsprograms supporting these extensions have been in use since 1981.The 1k packet length (XMODEM-1k) described below may be used inconjunction with YMODEM Batch Protocol, or with single file transfersidentical to the XMODEM/CRC protocol except for minimal changes to support1k packets.Another extension is simply called the g option.  It provides maximumthroughput when used with end to end error correcting media, such as X.PCand error correcting modems, including the emerging 9600 bps units byElectronic Vaults and others.To complete this tome, Ward Christensen's original protocol document andJohn Byrns's CRC-16 document are included for reference.References to the MODEM or MODEM7 protocol have been changed to XMODEM toaccommodate the vernacular.  In Australia, it is properly called theChristensen Protocol.__________ 1. Available for IBM PC,XT,AT, Unix and XenixChapter 2X/YMODEM Protocol Reference      09-11-86                                42.1  Some Messages from the Pioneer#: 130940 S0/Communications 25-Apr-85  18:38:47Sb: my protocolFm: Ward Christensen 76703,302 (EDITED)To: allBe aware the article[2] DID quote me correctly in terms of the phraseslike "not robust", etc.It was a quick hack I threw together, very unplanned (like everything Ido), to satisfy a personal need to communicate with "some other" people.ONLY the fact that it was done in 8/77, and that I put it in the publicdomain immediately, made it become the standard that it is.I think its time for me to(1) document it; (people call me and say "my product is going to includeit - what can I 'reference'", or "I'm writing a paper on it, what do I putin the bibliography") and(2) propose an "incremental extension" to it, which might take "exactly"the form of Chuck Forsberg's YAM protocol.  He wrote YAM in C for CP/M andput it in the public domain, and wrote a batch protocol for Unix[3] calledrb and sb (receive batch, send batch), which was basically XMODEM with   (a) a record 0 containing filename date time and size   (b) a 1K block size option   (c) CRC-16.He did some clever programming to detect false ACK or EOT, but basicallyleft them the same.People who suggest I make SIGNIFICANT changes to the protocol, such as"full duplex", "multiple outstanding blocks", "multiple destinations", etcetc don't understand that the incredible simplicity of the protocol is oneof the reasons it survived to this day in as many machines and programs asit may be found in!Consider the PC-NET group back in '77 or so - documenting to beat the band- THEY had a protocol, but it was "extremely complex", because it tried tobe "all things to all people" - i.e. send binary files on a 7-bit system,etc.  I was not that "benevolent". I (emphasize > I < ) had an 8-bit UART,__________ 2. Infoworld April 29 p. 16 3. VAX/VMS versions of these programs are also available.Chapter 2X/YMODEM Protocol Reference      09-11-86                                5so "my protocol was an 8-bit protocol", and I would just say "sorry" topeople who were held back by 7-bit limitations.  ...Block size: Chuck Forsberg created an extension of my protocol, calledYAM, which is also supported via his public domain programs for UNIXcalled rb and sb - receive batch and send batch.  They cleverly send a"block 0" which contains the filename, date, time, and size.Unfortunately, its UNIX style, and is a bit weird[4] - octal numbers, etc.BUT, it is a nice way to overcome the kludgy "echo the chars of the name"introduced with MODEM7.  Further, chuck uses CRC-16 and optional 1Kblocks.  Thus the record 0, 1K, and CRC, make it a "pretty slick newprotocol" which is not significantly different from my own.Also, there is a catchy name - YMODEM.  That means to some that it is the"next thing after XMODEM", and to others that it is the Y(am)MODEMprotocol.  I don't want to emphasize that too much - out of fear thatother mfgrs might think it is a "competitive" protocol, rather than an"unaffiliated" protocol.  Chuck is currently selling a much-enhancedversion of his CP/M-80 C program YAM, calling it Professional Yam, and itsfor the PC - I'm using it right now.  VERY slick!  32K capture buffer,script, scrolling, previously captured text search, plus built-in commandsfor just about everything - directory (sorted every which way), XMODEM,YMODEM, KERMIT, and ASCII file upload/download, etc.  You can program itto "behave" with most any system - for example when trying a number forCIS it detects the "busy" string back from the modem and substitutes adiff phone # into the dialing string and branches back to try it.3.  XMODEM PROTOCOL ENHANCEMENTSThis chapter discusses the protocol extensions to Ward Christensen's 1982XMODEM protocol description document.The original document recommends the user be asked whether to continuetrying or abort after 10 retries.  Most programs no longer ask theoperator whether he wishes to keep retrying.  Virtually all correctableerrors are corrected within the first few retransmissions.  If the line isso bad that ten attempts are insufficient, there is a significant dangerof undetected errors.  If the connection is that bad, it's better toredial for a better connection, or mail a floppy disk.__________ 4. The file length, time, and file mode are optional.  The pathname and    file length may be sent alone if desired.Chapter 3                                     XMODEM Protocol EnhancementsX/YMODEM Protocol Reference      09-11-86                                63.1  Graceful AbortThe YAM and Professional-YAM X/YMODEM routines recognize a sequence of twoconsecutive CAN (Hex 18) characters without modem errors (overrun,framing, etc.) as a transfer abort command.  This sequence is recognizedwhen YAM is waiting for the beginning

of a packet or for an acknowledge toone that has been sent.  The check for two consecutive CAN charactersvirtually eliminates the possibility of a line hit aborting the transfer.YAM sends eight CAN characters when it aborts an XMODEM, YMODEM, or ZMODEMprotocol file transfer.  Pro-YAM then sends eight backspaces to delete theCAN characters from the remote's keyboard input buffer, in case the remotehad already aborted the transfer and was awaiting a keyboarded command.3.2  CRC-16 OptionThe XMODEM protocol uses an optional two character CRC-16 instead of theone character arithmetic checksum used by the original protocol and bymost commercial implementations.  CRC-16 guarantees detection of allsingle and double bit errors,  all errors with an odd number of errorbits, all burst errors of length 16 or less, 99.9969% of all 17-bit errorbursts, and 99.9984 per cent of all possible longer error bursts.  Bycontrast, a double bit error, or a burst error of 9 bits or more can sneakpast the XMODEM protocol arithmetic checksum.The XMODEM/CRC protocol is similar to the XMODEM protocol, except that thereceiver specifies CRC-16 by sending C (Hex 43) instead of NAK whenrequesting the FIRST packet.  A two byte CRC is sent in place of the onebyte arithmetic checksum.YAM's c option to the r command enables CRC-16 in single file reception,corresponding to the original implementation in the MODEM7 seriesprograms.  This remains the default because many commercial communicationsprograms and bulletin board systems still do not support CRC-16,especially those written in Basic or Pascal.XMODEM protocol with CRC is accurate provided both sender and receiverboth report a successful transmission.  The protocol is robust in thepresence of characters lost by buffer overloading on timesharing systems.The single character ACK/NAK responses generated by the receiving programadapt well to split speed modems, where the reverse channel is limited toten per cent or less of the main channel's speed.XMODEM and YMODEM are half duplex protocols which do not attempt totransmit information and control signals in both directions at the sametime.  This avoids buffer overrun problems that have been reported byusers attempting to exploit full duplex asynchronous file transferprotocols such as Blast.Professional-YAM adds several proprietary logic enhancements to XMODEM'sChapter 3                                     XMODEM Protocol EnhancementsX/YMODEM Protocol Reference      09-11-86                                7error detection and recovery.  These com ideal conditions.3.3  XMODEM-1k 1024 Byte PacketThe choice to use 1024 byte packets is expressed to the sending program onits command line or selection menu.[1] 1024 byte packets improvethroughput in many applications, but some environments cannot accept 1024byte bursts, especially minicomuters running 19.2kb ports.An STX (02) replaces the SOH (01) at the beginning of the transmittedblock to notify the receiver of the longer packet length.  The transmittedpacket contains 1024 bytes of data.  The receiver should be able to acceptany mixture of 128 and 1024 byte packets.  The packet number isincremented by one for each packet regardless of the packet length.The sender must not change between 128 and 1024 byte packet lengths if ithas not received a valid ACK for the current packet.  Failure to observethis restriction allows certain transmission errors to pass undetected.If 1024 byte packets are being used, it is possible for a file to "grow"up to the next multiple of 1024 bytes.  This does not waste disk space ifthe allocation granularity is 1k or greater.  With YMODEM batchtransmission, the optional file length transmitted in the file name packetallows the receiver to discard the padding, preserving the exact filelength and contents.1024 byte packets may be used with batch file transmission or with singlefile transmission.  CRC-16 should be used with the k opt################################################################################################################################fer, then issue an informativediagnostic message if the receiver requests checksum instead of CRC-16.Under no circumstances may a sending program use CRC-16 unless thereceiver commands CRC-16.4.  YMODEM Batch File TransmissionThe YMODEM Batch protocol is an extension to the XMODEM/CRC protocol thatallows 0 or more files to be transmitted with a single command.  (Zerofiles may be sent if none of the requested files is accessible.) Thedesign approach of the YMODEM Batch protocol is to use the normal routinesfor sending and receiving XMODEM packets in a layered fashion similar to__________ 1. See "KMD/IMP Exceptions to YMODEM" below.Chapter 4                                     XMODEM Protocol EnhancementsX/YMODEM Protocol Reference      09-11-86                                8          Figure 1.  1024 byte Packets          SENDER                                  RECEIVER                                                  "s -k foo.bar"          "foo.bar open x.x minutes"                                                  C          STX 01 FE Data[1024] CRC CRC                                                  ACK          STX 02 FD Data[1024] CRC CRC                                                  ACK          STX 03 FC Data[1000] CPMEOF[24] CRC CRC                                                  ACK          EOT                                                  ACK          Figure 2.  Mixed 1024 and 128 byte Packets          SENDER                                  RECEIVER                                                  "s -k foo.bar"          "foo.bar open x.x minutes"                                                  C          STX 01 FE Data[1024] CRC CRC                                                  ACK          STX 02 FD Data[1024] CRC CRC                                                  ACK          SOH 03 FC Data[128] CRC CRC                                                  ACK          SOH 04 FB Data[100] CPMEOF[28] CRC CRC                                                  ACK          EOT                                                  ACKpacket switching methods.Why was it necessary to design a new batch protocol when one alreadyexisted in MODEM7?[1] The batch file mode used by MODEM7 is unsuitablebecause it does not permit full pathnames, file length, file date, orother attribute information to be transmitted.  Such a restrictive design,hastily implemented with only CP/M in mind, would not have permittedextensions to current areas of personal computing such as Unix, DOS, andobject oriented systems.  In addition, the MODEM7 batch file mode issomewhat susceptible to transmission impairments.__________ 1. The MODEM7 batch protocol transmitted CP/M FCB bytes f1...f8 and    t1...t3 one character at a time.  The receiver echoed these bytes as    received, one at a time.Chapter 4                                     XMODEM Protocol EnhancementsX/YMODEM Protocol Reference      09-11-86                                9As in the case of single a file transfer, the receiver initiates batchfile transmission by sending a "C" character (for CRC-16).The sender opens the first file and sends packet number 0 with thefollowing information.[2]Only the pathname (file name) part is required for batch transfers.To maintain upwards compatibility, all unused bytes in packet 0 must beset to null.Pathname The pathname (conventionally, the file name) is sent as a null     terminated ASCII string.  This is the filename format used by the     handle oriented MSDOS(TM) functions and C library fopen functions.     An assembly language example follows:                              DB      'foo.bar',0     No spaces are included iYAM's f option to  send the full pathname.  The source drive     (A:, B:, etc.) is never sent.     Filename Considerations:        + File names should be translated to lower case unless the sending          system supports upper/lower case file names.  This is a          convenience for users of systems (such as Unix) which store          filenames in upper and lower case.        + The receiver should accomm################################################################################################################################ be acceptable      to both the sender and receiving          operating systems.     If directories are included, they are delimited by /; i.e.,     "subdir/foo" is acceptable, "subdir\foo" is not.Length The file length and each of the succeeding fields are optional.[3]     The length field is stored in the packet as a decimal string counting     the number of data bytes in the file.  The file length does not     include any CPMEOF (^Z) or other garbage characters used to pad the     last packet.__________ 2. Only the data part of the packet is described here. 3. Fields may not be skipped.Chapter 4                                     XMODEM Protocol EnhancementsX/YMODEM Protocol Reference      09-11-86                               10     If the file being transmitted is growing during transmission, the     length field should be set to at least the final expected file     length, or not sent.     The receiver stor Date The mod date is optional, and       the filename and length     may be sent without requiring the mod date to be sent.     Iff the modification date is sent, a single space separates the     modification date from the file length.     The mod date is sent as an octal number giving the time the contents     of the file were last changed, measured in seconds from Jan 1 1970     Universal Coordinated Time (GMT).  A date of 0 implies

the     modification date is unknown and should be left as the date the file     is received.     This standard format was chosen to eliminate ambiguities arising from     transfers between different time zones.     Two Microsoft blunders complicate the use of modification dates in     file transfers with MSDOS(TM) systems.  The first is the lack of     timezone standardization in MS-DOS.  A file's creation time can not     be known unless the timezone of the system that wrote the file[4] is     known.  Unix solved this problem (for planet Earth, anyway) by     stamping files with Universal Time (GMT).  Microsoft would have to     include the timezone of origin in the directory entries, but does     not.  Professional-YAM gets around this problem by using the z     parameter which is set to the number of minutes local time lags GMT.     For files known to originate from a different timezone, the -zT     option may be used to specify T as  DOS.  Since some backup schemes used with DOS rely     on the file     creation date to select files to be copied to the archive, back-     dating the file modification date could interfere with the safety of     the transferred files.  For this reason, Pro-YAM does not modify the     date of received files with the header information unless the d     numeric parameter is non zeX/YMODEM Protocol Reference      09-11-86                               11Mode Iff the file mode is sent, a single space separates the file mode     from the modification date.  The file mode is stored as an octal     string.  Unless the file originated from a Unix system, the file mode     is set to 0.  rb(1) checks the file mode for the 0x8000 bit which     indicates a Unix type regular file.  Fil################################################################################################################################e conventions.  Such files are not translated     in any way.Serial Number Iff the serial number is sent, a single space separates the     serial number from the file mode.  The serial number of the     transmitting program is stored as an octal string.  Programs which do     not have a serial number should omit this field, or set it to 0.  The     receiver's use of this field is optional.Other Fields YMODEM was designed to allow additional header fields to be     added as above without creating compatibility problems with older     YMODEM programs.  Please contact Omen Technology if other fields are     needed for special application requirements.The rest of the packet is set to nulls.  This is essential to preserveupward compatibility.[5]If the filename packet is received with a CRC or other error, arestrnsmission is requested.  After the filename packet has been received,it is ACK'ed if the write open is successful.  If the file cannot beopened for writing, the receiver cancels the transfer with CAN charactersas described above.The receiver then initiates transfer of the file contents according to thestandard XMODEM/CRC protocol.After the file contents have been transmitted, the receiver again asks forthe next pathname.  Transmission of a null pathname terminates batch filetransmission.  Note that transmission of no files is not necessarily anerror.  This is possible if none of the files requested of the sendercould be opened for reading.In batch transmission, the receiver automatically requests CRC-16.The Unix programs sz(1) and rz(1) included in the source code file__________ 5. If, perchance, this information extends beyond 128 bytes (possible    with Unix 4.2 BSD extended file names), the packet should be sent as a    1k packet as described above.Chapter 4                                     XMODEM Protocol EnhancementsX/YMODEM Protocol Reference      09-11-86                               12RZSZ[12].SHQ (rzsz1.sh, rzsz2.sh) should answer other questions aboutYMODEM batch protocol.          Figure 3.  YMODEM Batch Transmission Session          SENDER                                  RECEIVER                                                  "sb foo.*<CR>"          "sending in batch mode etc."                                                  C (command:rb)          SOH 00 FF foo.c NUL[123] CRC CRC                                                  ACK                                                  C          SOH 01 FE Data[128] CRC CRC                                                  ACK          STX 02 FD Data[1024] CRC CRC                                                  ACK          SOH 03 FC Data[128] CRC CRC                                                  ACK          SOH 04 FB Data[100] CPMEOF[28] CRC CRC                                                  ACK          EOT                                                  NAK          EOT                                                  ACK                                                  C          SOH 00 FF NUL[128] CRC CRC                                                  ACK       Figure 4.  YMODEM Filename packet transmitted by sz       -rw-r--r--  6347 Jun 17 1984 20:34 bbcsched.txt       00 0100FF62 62637363 6865642E 74787400   |...bbcsched.txt.|       10 36333437 20333331 34373432 35313320   |6347 3314742513 |       20 31303036 34340000 00000000 00000000   |100644..........|       30 00000000 00000000 00000000 00000000       80 000000CA 56Chapter 4                                     XMODEM Protocol EnhancementsX/YMODEM Protocol Reference      09-11-86                               13      Figure 5.  YMODEM Header Information used by various programs_____________________________________________________________________| Program   | Batch | Length | Date | Mode | S/N | 1k-Blk | g-Option ||___________|_______|________|______|______|_____|________|__________||Unix rz/sz | yes   | yes    | yes  | yes  | no  | yes    | sb only  ||___________|_______|________|______|______|_____|________|__________||VMS rb/sb  | yes   | yes    | no   | no   | no  | yes    | no       ||___________|_______|________|______|______|_____|________|__________||Pro-YAM    | yes   | yes    | yes  | no   | yes | yes    | yes      ||___________|_______|________|______|______|_____|________|__________||CP/M YAM   | yes   | no     | no   | no   | no  | yes    | no       ||___________|_______|________|______|______|_____|________|__________||KMD/IMP    | yes   | ?      | no   | no   | no  | yes    | no       ||___________|_______|________|______|______|_____|________|__________|4.1  KMD/IMP Exceptions to YMODEMKMD and IMP character sequence emitted by the receiver (CK) toautomatically trigger the use of 1024 byte packets as an alternative tospecifying this option on this command line.  Although this two charactersequence works well on single process micros in direct communication,timesharing systems and packet switched networks can separate thesuccessive characters by several seconds, rendering this methodunreliable.Sending programs may detect the CK sequence if the operating enviornmentdoes not preclude reliable implementation.Receiving programs should retain the option of sending the standard YMODEMC (not CK) trigger with the standard 10 second timeout to insurecompatibility with newer forms of telecommunications technology.5.  YMODEM-g File TransmissionDeveloping technology is providing phone line data transmission at everhigher speeds using very specialized techniques.  These high speed modems,as well as session protocols such as X.PC, provide high speed, error freecommunications at the expense of considerably increased delay time.This delay time is moderate compared to human interactions, but itcripples the throughput of most error correcting protocols.The g option to YMODEM has proven effective under these circumstances.The g option is driven by the receiver, which initiates the batch transferby transmitting a G instead of C.  When the sender recognizes the G, itbypasses the usual wait for an ACK to each transmitted packet, sendingsucceeding packets at full speed, subject to XOFF/XON or other flowcontrol exerted by the medium.Chapter 5                                     XMODEM Protocol EnhancementsX/YMODEM Protocol Reference      09-11-86                               14The sender expects an inital G to initiate the transmission of aparticular file, and also expects an ACK on the EOT sent at the end ofeach file.  This synchronization allows the receiver time to open andclose files as necessary.If an error is detected in a YMODEM-g transfer, the receiver aborts thetransfer with the multiple CAN abort sequence.  The ZMODEM protocol shouldbe used in applications that require both streaming throughput and errorrecovery.       Figure 6.  YMODEM-g Transmission Session       SENDER                                    RECEIVER                                                 "sb foo.*<CR>"       "sending in batch mode etc..."                                                 G (command:rb -g)       SOH 00 FF foo.c NUL[123] CRC CRC                                                 G       SOH 01 FE Data[128] CRC CRC       STX 02 FD Data[1024] CRC CRC       SOH 03 FC Data[128] CRC CRC       SOH 04 FB Data[100] CPMEhristensen.I will maintain a master copy of this.  Please pass on changes orsuggestions via CBBS/Chicago at (312) 545-8086, CBBS/CPMUG (312) 849-1132or by voice at (312) 849-6279.6.1  Definitions  <soh> 01H  <eot> 04H  <ack> 06H  <nak> 15H  <can> 18H  <C>   43HChapter 6                                         Xmodem Protocol OverviewX/YMODEM Protocol Reference      09-11-86                               156.2  Tr################################################################################################################################he

contents of   the data beingtransmitted.  No control characters are looked for in the 128-byte datamessages.  Absolutely any kind of data may be sent - binary, ASCII, etc.The protocol has not formally been adopted to a 7-bit environment for thetransmission of ASCII-only (or unpacked-hex) data , although it could besimply by having both ends agree to AND the protocol-dependent data with7F hex before validating it.  I specifically am referring to the checksum,and the block numbers and their ones- complement.Those wishing to maintain compatibility of the CP/M file structure, i.e.to allow modemming ASCII files to or from CP/M systems should follow thisdata format:   + ASCII tabs used (09H); tabs set every 8.   + Lines terminated by CR/LF (0DH 0AH)   + End-of-file indicated by ^Z, 1AH.  (one or more)   + Data is variable length, i.e. should be considered a continuous     stream of data bytes, broken into 128-byte chunks purely for the     purpose of transmission.   + A CP/M "peculiarity": If the data ends exactly on a 128-byte     boundary, i.e. CR in 127, and LF in 128, a subsequent sector     containing the ^Z EOF character(s) is optional, but is preferred.     Some utilities or user programs still do not handle EOF without ^Zs.   + The last block sent is no different from others, i.e.  there is no     "short block".              Figure 7.  XMODEM Message Block Level ProtocolEach block of the transfer looks like:      <SOH><blk #><255-blk #><--128 data bytes--><cksum>in which:<SOH>         = 01 hex<blk #>       = binary number, starts at 01 increments by 1, and                wraps 0FFH to 00H (not to 01)<255-blk #>   = blk # after going thru 8080 "CMA" instr, i.e.                each bit complemented in the 8-bit block number.                Formally, this is the "ones complement".<cksum>       = the sum of the data bytes only.  Toss any carry.Chapter 6                                         Xmodem Protocol OverviewX/YMODEM Protocol Reference      09-11-86                               166.3  File Level Protocol6.3.1  Common_to_Both_Sender_and_ReceiverAll errors are retried 10 times.  For versions running with an operator(i.e. NOT with XMODEM), a message is typed after 10 errors asking theoperator whether to "retry or quit".Some versions of the protocol use <can>, ASCII ^X, to cancel transmission.This was never adopted as a standard, as having a single "abort" charactermakes the transmission susceptible to false termination due to an <ack><nak> or <soh> being corrupted into a <can> and aborting transmission.The protocol may be considered "receiver driven", that is, the sender neednot automatically re-transmit, although it does in the currentimplementations.6.3.2  Receive_Program_ConsiderationsThe receiver has a 10-second timeout.  It sends a <nak> every time ittimes out.  The receiver's first timeout, which sends a <nak>, signals thetransmitter to start.  Optionally, the receiver could send a <nak>immediately, in case the sender was ready.  This would save the initial 10second timeout.  However, the receiver MUST continue to timeout every 10seconds in case the sender wasn't ready.Once into a receiving a block, the receiver goes into a one-second timeoutfor each character and the checksum.  If the receiver wishes to <nak> ablock for any reason (invalid header, timeout receiving data), it mustwait for the line to clear.  See "programming tips" for ideasSynchronizing:  If a valid block number is received, it will be: 1) t################################################################################################################################K, and onlyindicates that the receivers <ack> got glitched, and the sender re-transmitted; 3) any other block number indicates a fatal loss ofsynchronization, such as the rare case of the sender getting a line-glitchthat looked like an <ack>.  Abort the transmission, sending a <can>6.3.3  Sending_program_considerationsWhile waiting for transmission to begin, the sender has only a single verylong timeout, say one minute.  In the current protocol, the sender has a10 second timeout before retrying.  I suggest NOT doing this, and lettingthe protocol be completely receiver-driven.  This will be compatible withexisting programs.When the sender has no more data, it sends an <eot>, and awaits an <ack>,resending the <eot> if it doesn't get one.  Again, the protocol could bereceiver-driven, with the sender only having the high-level 1-minutetimeout to abort.Chapter 6                                       block message.  It includesthe two most common line hits - a garbaged block, and an <ack> replygetting garbaged.  <xx> represents the checksum byte.              Figure 8.  Data flow including Error RecoverySENDER                                  RECEIVER                              times out after 10 seconds,                              <---              <nak><soh> 01 FE -data- <xx>       --->                              <---              <ack><soh> 02 FD -data- xx         --->       (data gets line hit)                              <---              <nak><soh> 02 FD -data- xx         --->                              <---              <ack><soh> 03 FC -data- xx         --->(ack gets garbaged)           <---              <ack><soh> 03 FC -data- xx         --->              <ack><eot>                         --->                              <---       <anything except ack><eot>                         --->                              <---              <ack>(finished)6.4  Programming Tips   + The character-receive subroutine should be called with a parameter     specifying the number of seconds to wait.  The receiver should first     call it with a time of 10, then <nak> and try again, 10 times.     After receiving the <soh>, the receiver should call the character     receive subroutine with a 1-second timeout, for the remainder of the     message and the <cksum>.  Since they are sent as a continuous stream,     timing out of this implies a serious like glitch that caused, say,     127 characters to be seen instead of 128.   + When the receiver wishes to <nak>, it should call a "PURGE"     subroutine, to wait for the line to clear.  Recall the sender tosses     any characters in its UART buffer immediately upon completing sending     a block, to ensure no glitches were mis- interpreted.     The most common technique is for "PURGE" to call the character     receive subroutine, specifying a 1-second timeout,[1] and looping     back to PURGE until a timeout occurs.  The <nak> is then sent,     ensuring the other end will see it.__________ 1. These times should be adjusted for use with timesharing systems.Chapter 6                                         Xmodem Protocol OverviewX/YMODEM Protocol Reference      09-11-86                               18   + You may wish to add code recommended by John Mahr to your character     receive routine - to set an error flag if the UART shows framing     error, or overrun.  This will help catch a few more glitches - the     most common of which is a hit in the high bits of the byte in two     consecutive bytes.  The <cksum> comes out OK since counting in 1-byte     produces the same result of adding 80H + 80H as with adding 00H +     00H.7.  XMODEM/CRC Overview1/13/85 by John Byrns -- CRC option.Please pass on any reports of errors in this document or suggestions forimprovement to me via Ward's/CBBS at (312) 849-1132, or by voice at (312)885-1105.The CRC used in the Modem Protocol is an alternate form of block checkwhich provides more robust error detection than the original checksum.Andrew S. Tanenbaum says in his book, Computer Networks, that the CRC-CCITT used by the Modem Protocol will detect all single and double biterrors, all errors with an odd number of bits, all burst errors of length16 or less, 99.997% of 17-bit error bursts, and 99.998% of 18-bit andlonger bursts.The changes to the Modem Protocol to replace the checksum with the CRC arestraight forward. If that were all that we did we would not be able tocommunicate between a program using the old checksum protocol and oneusing the new CRC protocol. An initial handshake was added to solve thisproblem. The handshake allows a receiving program with CRC capability todetermine whether the sending program supports the CRC option, and toswitch it to CRC mode if it does. This handshake is designed so that itwill work properly with programs which implement only the originalprotocol. A description of this handshake is presented in section 10.            Figure 9.  Message Block Level Protocol, CRC modeEach block of the transfer in CRC mode looks like:     <SOH><blk #><255-blk #><--128 data bytes--><CRC hi><CRC lo>in which:<SOH>        = 01 hex<blk #>      = binary number, starts at 01 increments by 1, and               wraps 0FFH to 00H (not to 01)<255-blk #>  = ones complement of blk #.<CRC hi>     = byte containing the 8 hi order coefficients of the CRC.<CRC lo>     = byte containing the 8 lo order coefficients of the CRC.Chapter 7                                         Xmodem Protocol OverviewX/YMODEM Protocol Reference      09-11-86                               197.1  CRC Calculation7.1.1  Formal_DefinitionTo calculate the 16 bit CRC the message bits are considered to be thecoefficients of a polynomial. This message polynomial is first multipliedby X^16 and then divided by the generator polynomial (X^16 + X^12 + X^5 +1) using modulo two arithmetic. The remainder left after the division isthe desired CRC. Since a message block in the Modem Protocol is 128 bytesor 1024 bits, the message polynomial will be of order X^1023. The hi orderbit

of the first byte of the message block is the coefficient of X^1023 inthe message polynomial.  The lo order bit of the last byte of the messageblock is the coefficient of X^0 in the messag code for theseprograms (contained in RZSZ1.SHQ and RZSZ2.SHQ) for usage.  A fast tabledriven macro is also included in this file./* update CRC */unsigned shortupdcrc(c, crc)register c;register unsigned crc;{        register count;        for (count=8; --count>=0;) {                if (crc & 0x8000) {                        crc <<= 1;                        crc += (((c<<=1) & 0400)  !=  0);                        crc ^= 0x1021;                }                else {                        crc <<= 1;                        crc += (((c<<=1) & 0400)  !=  0);                }        }        return crc;}7.2  CRC File Level Protocol Changes7.2.1  Common_to_Both_Sender_and_ReceiverThe only change to the File Level Protocol for the CRC option is theinitial handshake which is used to determine if both the sending and thereceiving programs support the CRC mode. All Modem Programs should supportthe checksum mode for compatibility with older versions.  A receivingprogram that wishes to receive in CRC mode implements the mode settinghandshake by sending a <C> in place of the initial <nak>.  If the sendingprogram supports CRC mode it will recognize the <C> and will set itselfinto CRC mode, and respond by sending the first block as if a <nak> hadbeen received. If the sending program does not support CRC mode it willChapter 7                                         Xmodem Protocol OverviewX/YMODEM Protocol Reference      09-11-86                               20not respond to the <C> at all. After the receiver has sent the <C> it willwait up to 3 seconds for the <soh> that starts the first block. RC mode. If no  <soh> isreceived within 3 seconds the receiver will switch to checksum mode, senda <nak>, and proceed in checksum mode. If the receiver wishes to usechecksum mode it should send an initial <nak> and the sending programshould respond to the <nak> as defined in the original Modem Protocol.After the mode has been set by the initial <C> or <nak> the protocolfollows the original Modem Protocol and is identical whether the checksumor CRC is being used.7.2.2  Receive_Program_ConsiderationsThere are at least 4 things that can go wrong with the mode settinghandshake.  1.  the initial <C> can be garbled or lost.  2.  the initial <soh> can be garbled.  3.  the initial <C> can be changed to a <nak>.  4.  the initial <nak> from a receiver which wants to receive in checksum      can be changed to a <C>.The first problem can be solved if the receiver sends a second <C> afterit times out the first time. This process can be repeated several times.It must not be repeated too many times before sending a <nak> andswitching to checksum mode or a sending program without CRC support maytime out and abort. Repeating the <C> will also fix the second problem ifthe sending program cooperates by responding as if a <nak> were receivedinstead of ignoring the extra <C>.It is possible to fix problems 3 and 4 but probably not worth the troublesince they will occur very infrequently. They could be fixed by switchingmodes in either the sending or the receiving program after a large numberof successive <nak>s. This solution would risk other problems however.7.2.3  Sending_Program_ConsiderationsThe sending program should start in the checksum mode. This will insurecompatibility with checksum only receiving programs. Anytime a <C> isreceived before the first <nak> or <ack> the sending program should setitself into CRC mode and respond as if a <nak> were received. The sendershould respond to additional <C>s as if they were <nak>s until the first<ack> is received. This will assist the receiving program in determiningthe correct mode when the <soh> is lost or garbled. After the first <ack>is received the sending program should ignore <C>s.Chapter 7                                         Xmodem Protocol OverviewX/YMODEM Protocol Reference      09-11-86                               217.3  Data Flow Examples with CRC OptionHere is a data flow example for the case where the receiver requeststransmission in the CRC m checksum      byte.      Figure 11.  Data Flow: Receiver has CRC Option, Sender Doesn'tSENDER                                        RECEIVER                        <---                <C>                                times out after 3 seconds,                        <---                <C>                                times out after 3 seconds,                        <---                <C>                                times out after 3 seconds,                        <---                <C>                                times out after 3 seconds,                        <---                <nak><soh> 01 FE -data- <xx> --->                        <---                <ack><soh> 02 FD -data- <xx> --->        (data gets line hit)                        <---                <nak><soh> 02 FD -data- <xx> --->                        <---                <ack><soh> 03 FC -data- <xx> --->   (ack gets garbaged)  <---                <ack>                                times out after 10 seconds,                        <---                <nak><soh> 03 FC -data- <xx> --->                        <---                <ack><eot>                   --->                        <---                <ack>Here is a data flow example for the case where the receiver requeststransmission in the CRC mode and the sender supports the CRC option.  Thisexample also includes various transmission errors.  <xxxx> represents the2 CRC bytes.Chapter 7                                         Xmodem Protocol OverviewX/YMODEM Protocol Reference      09-11-86                               22           Figure 12.  Receiver and Sender Both have CRC OptionSENDER                                       RECEIVER                          <---                 <C><soh> 01 FE -data- <xxxx> --->                          <---                 <ack><soh> 02 FD -data- <xxxx> --->         (data gets line hit)                          <---                 <nak><soh> 02 FD -data- <xxxx> --->                          <---                 <ack><soh> 03 FC -data- <xxxx> --->(ack gets garbaged)       <---                 <ack>                                     times out after 10 seconds,                          <---                 <nak><soh> 03 FC -data- <xxxx> --->                          <---                 <ack><eot>                     --->                          <---                 <ack>8.  MORE INFORMATIONMore information may be obtained by calling Telegodzilla at 503-621-3746.Hit RETURNs for baud rate detection.Speed detection is automatic for 300,1200, and 2400 bps.A version this file with boldface, underlining, and superscripts forprinting on Epson or Gemini printers is available on Telegodzilla as"YMODEME.DOC" or "YMODEME.DQCThe following L.sys line calls Telegodzilla (Pro-YAM in host operation).Telegodzilla waits for carriage returns to determine the incoming speed.If none is detected, 1200 bps is assumed and a greeting is displayed.In response to "Name Please:" uucico gives the Pro-YAM "link" command as auser name.  The password (Giznoid) controls access to the Xenix systemconnected to the IBM PC's other serial port.  Communications betweenPro-YAM and Xenix use 9600 bps; YAM converts this to the caller's speed.Finally, the calling uucico logs in as uucp.omen Any ACU 1200 1-503-621-3746 se:--se: link ord: Giznoid in:--in: uucpContact omen!caf if you wish the troff source files.Chapter 8                                         Xmodem Protocol OverviewX/YMODEM Protocol Reference      09-11-86                               239.  Document RevisionsThe September 11 1986 edition clarifies nomenclature and some minorpoints.  The April 15 1986 edition clarifies some points concerning CRCcalculations and spaces in the header.10.  YMODEM ProgramsA demonstration version of Professional-YAM is available as ZMODEM.ARCThis may be used to test YMODEM amd ZMODEM implementations.Unix programs supporting the YMODEM protocol are available on Telegodzillaas RZSZ1.SHQ and RZSZ2.SHQ (SQueezed shell archives).  Most Unix likesystems are supported, including V7, Xenix, Sys III, 4.2 BSD, SYS V,Idris, Coherent, and Regulus.A version for VAX-VMS is available in VRBSB.SHQ.Irv Hoff has added YMODEM 1k packets and basic YMODEM batch transfers tothe KMD and IMP series programs, which replace the XMODEM andMODEM7/MDM7xx series respectively.  Overlays are available for a widevariety of CP/M systems.Questions about YMODEM, the Professional-YAM communications program, andrequests for evaluation copies may be directed to:     Chuck Forsberg     Omen Technology Inc     17505-V Sauvie Island Roaeed!omen!caf     Compuserve: 70007,2304     Source: TCE022Chapter 10                                        Xmodem Protocol Overview                                 CONTENTS 1.  ROSETTA STONE....................................................   2 2.  YET ANOTHER PROTOCOL?............................................   2     2.1  Some Messages from the Pioneer..............................   4 3.  XMODEM.....         6     3.2  CRC-16 Option...............................................   6     3.3  XMODEM-1k 1024 Byte Packet..................................   7 4.  YMODEM Batch File Transmission...................................   7     4.1  KMD/IMP Exceptions to YMODEM................................  13 5.  YMODEM-g File Transmission.......................................

13 6.  XMODEM PROTOCOL OVERVIEW.........................................  14     6.1  Definitions.................................................  14     6.2  Transmission Medium Level Protocol..........................  15     6.3  File Level Protocol.........................................  16     6.4  Programming Tips............................................  17 7.  XMODEM/CRC Overview..............................................  18     7.1  CRC Calculation.............................................  19     7.2  CRC File Level Protocol Changes.............................  19     7.3  Data Flow Examples with CRC Option..........................  21 8.  MORE INFORMATION.................................................  22 9.  Document Revisions...............................................  2310.  YMODEM Programs..................................................  23                                  - i -                             LIST OF FIGURES Figure 1.  1024 byte Packets.........................................   7 Figure 2.  Mixed 1024 and 128 byte Packets...........................   7 Figure 3.  YMODEM Batch Transmission Session.........................  12 Figure 4.  YMODEM Filename packet transmitted by sz..................  12 Figure 5.  YMODEM Header Information used by various programs........  13 Figure 6.  YMODEM-g Transmission Session.............................  14 Figure 7.  XMODEM Message Block Level Protocol.......................  15 Figure 8.  Data flow including Error Recovery........................  17 Figure 9.  Message Block Level Protocol, CRC mode....................  18Figure 10.  Example of CRC Calculation written in C...................  19Figure 11.  Data Flow: Receiver has CRC Option, Sender Doesn't........  21Figure 12.  Sender and Receiver both have CRC Option .................  22#########################################################################################################################################################